To help you move from the pilot phase to a commercial-scale operation of 120 raceways, I’ve outlined the primary infrastructure requirements and the estimated power consumption. This data is essential for your business plan to demonstrate a clear understanding of both CAPEX (Capital Expenditure) and OPEX (Operating Expenditure).

## 1. Line-Item Equipment List (Estimates)

This list assumes a central pump station feeding a manifold that distributes water across your 120-unit array.

| Component | Specification | Quantity | Purpose |
| --- | --- | --- | --- |
| Main Intake Pumps | 5.5kW - 7.5kW Mixed Flow (IP66) | 2 Units | 1 Duty, 1 Standby (Redundancy). |
| VFD Controllers | 7.5kW Inverter (IP66 rated) | 2 Units | Speed control for exchange rate adjustment. |
| Main Trunk Line | 250mm - 315mm HDPE (PN10) | ~150m | Main artery to move $180\text{ m}^3\text{/hr}$. |
| Distribution Manifold | 160mm - 200mm PVC-U | 4-6 Lines | Feeds rows of 20-30 raceways each. |
| Raceway Inlets | 40mm Ball Valve + Flow Meter | 120 Units | Precise $25\text{ L/min}$ control per tank. |
| Paddlewheel Motors | 0.37kW Brushless DC | 120 Units | Internal circulation and "Cleaning Pulse." |
| Control Logic (PLC) | Centralized Industrial Controller | 1 Unit | Automates the cleaning pulses and alarms. |

## 2. Power Consumption Estimate (OPEX)

Electricity is often the second-highest cost after feed in land-based aquaculture. For 120 raceways, here is your daily draw:

### Primary Intake Pumping

- Average Load: $5.5\text{ kW}$ (Running 24/7).

- Daily Consumption: $132\text{ kWh}$.

### Paddlewheels (The "Stirrers")

- Standard Mode (23 hrs): 120 units at $0.05\text{ kW}$ load = $138\text{ kWh}$.

- Cleaning Pulse (1 hr): 120 units at $0.25\text{ kW}$ load = $30\text{ kWh}$.

- Daily Consumption: $168\text{ kWh}$.

### Total Estimated Daily Draw: $300\text{ kWh}$

Business Plan Note: At an average Australian commercial rate of $0.25\text{ per kWh}$, your daily power cost would be approximately $75.00 ($27,375 per year).

## 3. Engineering Strategy for Fundraising

When presenting to investors or banks, emphasize these three "Efficiency Pillars" inherent in your design:

- Hydraulic Efficiency: By using large-diameter pipes (315mm), we minimize "friction loss," allowing us to use smaller, cheaper-to-run pumps to move the same volume of water.

- Decoupled Flow: We achieve high-velocity cleaning with small paddlewheel motors ($0.37\text{kW}$) rather than trying to achieve that velocity through massive water exchange, which would require $5\times$ more pumping power.

- Modular Scalability: The system is designed in "Banks of 20." This allows you to start month 1 with 20 raceways and scale up to 120 by month 7 without redesigning the primary intake.

## 4. Immediate Technical Consideration

With 120 raceways, you will generate a significant volume of "waste-heavy" water during the cleaning pulses.

- Engineering Tip: You need a Central Drainage Channel capable of handling a surge of water. I recommend a sloped concrete or fiberglass "V-trough" running between the rows of raceways, leading to a settling pond or mechanical drum filter before the water is discharged or recirculated.

### Suggested Next Step

Would you like me to draft a "Risk Mitigation" section for your manual? This would cover the protocols for a total power failure—specifically how to maintain dissolved oxygen in those 120 shallow tanks until the backup generator kicks in.