A computer vision program is available to the public. The program can be found at https://github.com/TheoBatik/urchinvision.

Key aspects of the computer vision program for measuring sea urchins:

- It is written in Python 3, using the OpenCV library.

- It applies a hue saturation value filter to remove most spines.

- It uses a Canny contour detection algorithm to obtain an edge map.

- It applies morphological operations to remove the remaining spines and smooth the image.

- It determines contours using topological structural analysis.

- It sets a minimum area bounding rectangle around the contour of objects identified in the image.

- It calculates a pixel-per-metric ratio using a reference object of known dimensions.

- It reduces distortion by choosing the smallest value between the vertical or horizontal width of an object as the primary measurement value.

The computer vision technology was found to be more precise and efficient when compared to measuring urchin diameter manually with callipers, and at least three times faster than the manual method because multiple urchins can be measured at once. The exact number of urchins the software could process is dependent on the image quality and processing power of the computer used. The computer vision protocol only requires one person, whereas the manual method requires an operator and a scribe, halving labour requirements. The study indicates that there is great potential to further develop the computer vision program.

Additionally, the source notes that training a custom object detection model requires a large dataset of images of Tripneustes gratilla, labelled by drawing bounding boxes around each urchin.