### Financial Estimate: Liquid O2 and Aeration Network (Marine RAS)

1. System Requirements

- Production Load: 7 cohorts of 50,000 T. gratilla (350,000 total animals onsite). Assuming a linear growth curve to 200g, the facility will hold an estimated standing biomass of roughly 30 to 35 tonnes at any given time.

- O2 Demand (RAS Specific): In a Marine RAS, oxygen is consumed not just by the urchins, but heavily by the nitrifying bacteria in the moving bed bioreactors (MBBR) or biofilters. Peak demand requires maintaining >6.5 mg/L DO across 5,400 sqm of tank space and filtration loops.

- Infrastructure Need: 1x 5,000L Liquid Oxygen (LOX) cryogenic storage vessel, high-capacity ambient vaporizers, main PVC/316 Stainless distribution manifold, oxygen cones/speece cones for dissolution into the RAS return lines, and regenerative blowers for biofilter aeration and emergency tank support.

2. CAPEX Breakdown (AUD)

- LOX 5,000L Storage Vessel (Concrete pad prep, safety bollards, and lease installation): $12,500

- High-Capacity Ambient Vaporizers & Dual Pressure Regulators: $6,800

- Main O2 Distribution Network & Oxygen Cones (for efficient RAS dissolution): $18,500

- Regenerative Blowers (x4, 3-phase, for MBBR aeration & tank degassing): $14,200

- Fine-pore Ceramic Diffusers (for biofilters and emergency tank drops): $8,000

- Optical DO Monitoring Sensors, automated solenoid valves, & SCADA integration: $22,000

- Installation, Pipefitting & Commissioning: $14,000

- Contingency (15%): $14,400

- Total Subsystem CAPEX: $110,400

3. OPEX Breakdown - Annual (AUD)

- LOX Bulk Delivery (Estimated 18 refills/year to Chinderah): $28,000

- LOX Tank Monthly Lease & Telemetry Fee: $4,200

- Electricity (Continuous blower operation for biofilters + monitoring): $14,500

- Routine Maintenance (Sensor cap replacements, blower servicing, valve checks): $4,500

- Direct Labor (0.15 FTE for daily DO calibration and system checks): $11,250

- Total Annual OPEX: $62,450

4. Key Assumptions

- Pricing assumes standard industrial gas (BOC/Coregas) delivery access to the Chinderah site.

- Electricity is calculated at regional commercial tariffs; blowers run 24/7 in an RAS environment to maintain biofilter health.

- Equipment depreciation lifespan: Diffusers/Sensors (3-5 years), Blowers/O2 Cones (10 years).

- The O2 cones provide 90%+ dissolution efficiency into the water stream before it enters the urchin tanks, minimizing off-gassing waste compared to dropping airstones directly into shallow grow-out trays.

Your master prompt is locked and loaded in my memory.

Whenever you are ready, just tell me the next subsystem you want to tackle. Would you like me to run the Grow-out Tank Infrastructure next, or do you want to calculate the Marine RAS Water Treatment (Biofiltration & Skimmers)?