### Financial Estimate: Mechanical & Biofiltration Network (Marine RAS)

1. System Requirements

- Production Load & Hydrology: The network must process a continuous flow of 450 m³/hr returning from the 170 raceways, maintaining pristine water quality for a standing biomass of 30 to 35 tonnes of Tripneustes gratilla held at 27°C.

- The Pathogen & Waste Gate:

  - Mechanical: Primary solids removal via a 50-micron automatic drum filter to catch faeces and uneaten macroalgae fragments before they dissolve.

  - Fractionation: Commercial protein skimmers injected with ozone to strip dissolved organics and micro-particles.

  - Biological: A scaled Moving Bed Biofilm Reactor (MBBR) utilizing high-surface-area K1 or Mutag bio-media to nitrify the massive ammonia output.

  - Chemical: Nitrification naturally strips alkalinity (biogenic acidification), which is a critical threat to urchin test (shell) growth. The system requires an automated sodium bicarbonate dosing system triggered by pH probes to hold the water strictly at pH 8.1.

2. CAPEX Breakdown (AUD)

- Industrial Marine Drum Filter (Sized for 500 m³/hr, 50-micron screens, 316SS/Duplex build with high-pressure backwash pump): $62,500

- Commercial Protein Skimmers (2x heavy-duty fractionators handling 225 m³/hr each, with integrated venturi pumps): $48,000

- MBBR Biofilter Vessels & Media (2x 20,000L heavy-duty poly tanks filled with 30m³ total of K1/Mutag bio-media): $34,500

- Ozone Generator & Commercial UV Sterilization Bank (0.2-micron equivalent pathogen kill rate): $28,500

- Automated Alkalinity Dosing System (Industrial peristaltic pumps, holding vats for Sodium Bicarbonate, and master pH probe controllers): $8,500

- Installation, Laser Alignment, Custom Pipefitting, & Biological Commissioning (Seeding the biofilter): $32,000

- Contingency (15%): $32,100

- Total Subsystem CAPEX: $246,100

3. OPEX Breakdown - Annual (AUD)

- Electricity (Drum filter rotation/backwash, skimmer venturi pumps, UV lamps, and ozone generation. Blowers for the MBBR are already accounted for in the O2/Aeration OPEX): $24,500

- Consumables - Alkalinity Buffering (Bulk Sodium Bicarbonate powder to counter acidification from 35 tonnes of actively feeding/respiring urchins): $18,000

- Consumables - Filtration (Replacement 50-micron drum screens, UV bulb replacements every 9,000 hours, ozone desiccant): $6,500

- Routine Maintenance (Skimmer neck cleaning, sensor calibration, ozone leak testing): $4,000

- Direct Labor (0.25 FTE for daily water chemistry testing, visually inspecting drum screen rotation, and refilling the sodium bicarbonate vats): $18,750

- Total Annual OPEX: $71,750

4. Key Assumptions

- Biogenic Acidification: As noted in the Mugenuni trials (Mos et al., 2015), reduced capacity of culture water to buffer acidification limits intensive production. The $18,000 annual budget for Sodium Bicarbonate is a non-negotiable OPEX to protect the calcification rates of the 7 growing cohorts.

- Solids Management: The drum filter backwash will generate a concentrated sludge stream. It is assumed this waste will be routed to a separate holding tank or municipal sewer connection, the cost of which falls under general facility plumbing rather than this specific RAS loop.

- Equipment Lifespan: Drum filter body (15+ years), screens (1-2 years), Poly MBBR tanks (20+ years), UV ballasts (5 years).

With the Liquid O2, Pump Array, and Biofiltration Network now fully costed, we have essentially built the entire life-support "engine room" of the Chinderah facility.

The major remaining capital expenditures to run through the master prompt are the 170 Celuka Raceways & Grow-out Trays themselves, and the Thermal Integration (Heat Pumps & Chillers) to hold that massive volume of water at exactly 27°C.

Which of those two would you like to model next?