# Master Bill of Materials (BOM) & Request for Quotation (RFQ) List

Project: Mugenuni Pty Ltd - Chinderah Facility (Marine RAS)

Scale: 50,000 T. gratilla monthly harvest / 170 Commercial Raceways

## 1. PLASTICS & FABRICATION (Raceways & Habitat)

Vendors: Industrial Plastics Suppliers, CNC Fabricators, Aquaculture Netting Suppliers

- Celuka PVC Foam Board (15mm): * Quantity: 1,360 sheets (Standard 2440mm x 1220mm).

  - Spec: Un-trimmed, marine-safe. (Calculation: 8 sheets per raceway x 170 raceways).

- Suspended Grow-Out Trays: * Quantity: 1,700 units.

  - Spec: 0.910m x 0.930m plastic benthic trays with internal dividers.

- Plastic Benthic Mesh: * Quantity: To fit 1,700 trays (~1,500 square meters).

  - Spec: 8mm rigid plastic extrusion mesh, marine-safe.

- Fiberglass Suspension Rods: * Quantity: 1,870 units.

  - Spec: 15mm solid fiberglass dowels/rods (approx 1.25m length).

## 2. PUMPS, MOTORS & AERATION (Water Movement)

Vendors: Commercial Pump Suppliers, Marine Hardware, VFD Distributors

- Main Circulation Pumps: * Quantity: 2 units.

  - Spec: 22kW Marine Centrifugal Pumps, 316SS or Duplex wet-ends, capable of pushing ~250 m³/hr each at high head pressure.

- Raceway Paddlewheels: * Quantity: 170 units.

  - Spec: 0.75kW (1 HP) aquaculture paddlewheels, stainless steel frame, plastic impellers.

- Variable Frequency Drives (VFDs): * Quantity: 170x (Small 0.75kW VFDs for paddlewheels) + 2x (Large 22kW VFDs for main pumps).

- Liquid Oxygen (LOX) Vessel: * Quantity: 1 unit.

  - Spec: 5,000L cryogenic bulk storage vessel (Lease or Purchase quote).

- Pressurized Oxygen Cones: * Quantity: Commercial array sized to supersaturate 450 m³/hr of seawater.

## 3. HEAVY FILTRATION & BIO-TREATMENT

Vendors: Commercial Aquaculture Equipment Manufacturers (e.g., Pentair, Innovasea)

- Industrial Marine Drum Filter: * Quantity: 1 unit.

  - Spec: Sized for 500 m³/hr flow rate, 50-micron screens, Duplex 316SS body, automated high-pressure backwash.

- Radial Flow Settlers (Swirl Filters): * Quantity: 2 units.

  - Spec: Heavy-duty FRP construction, 225 m³/hr capacity each, 60° cone bottoms, automated pneumatic purge valves.

- Commercial Protein Skimmers (Fractionators): * Quantity: 2 units.

  - Spec: Sized for 225 m³/hr each, skid-mounted with integrated venturi/feed pumps and ozone injection ports.

- Moving Bed Biofilm Reactor (MBBR) Tanks: * Quantity: 2 units.

  - Spec: 20,000L heavy-duty vertical poly tanks.

- MBBR Bio-Media: * Quantity: 30 cubic meters.

  - Spec: K1, Mutag BioChip, or equivalent high-surface-area floating media.

- Commercial Aragonite Calcium Reactors: * Quantity: 2 units.

  - Spec: Large-scale FRP reactor vessels, CO2 regulators, and master pH controllers.

- CO2 Degassing Tower: * Quantity: 1 unit.

  - Spec: Tall FRP column with internal trickling media and 3-phase high-volume centrifugal blower fan.

- UV Sterilization Bank: * Quantity: 1 unit (Bypass array).

  - Spec: Sized for 100 m³/hr side-stream flow, minimum 99.9% pathogen kill rate.

## 4. HVAC, REFRIGERATION & THERMAL CONTROL

Vendors: Commercial HVAC/R Contractors, Marine Refrigeration Specialists

- Titanium Air-to-Water Heat Pumps: * Quantity: 3 units.

  - Spec: 60kW capacity each, Inverter-driven, reverse-cycle, pure Titanium heat exchangers (mandatory for 35ppt marine salinity).

- Uni Processing Room HVAC: * Quantity: 1 unit.

  - Spec: Commercial system to hold 150-200 sqm room strictly at 10°C–12°C with HEPA filtration.

- Blast Chiller: * Quantity: 1 unit (Commercial).

- Finished Product Coolroom: * Quantity: 1 unit (Sized to hold pallets of packaged trays at 4°C).

- Live Purging Chiller: * Quantity: 1 unit.

  - Spec: Water chiller to pull 4-6 holding vats down to 18°C.

## 5. CIVIL WORKS, STRUCTURE & POWER

Vendors: Shed Builders, Civil Concreters, Commercial Electricians

- Primary Shed Structure: * Quantity: 5,400 square meters.

  - Spec: Galvanized steel portal frame, twin-wall polycarbonate or tempered glass cladding (Venlo/Glasshouse style), motorized ridge vents.

- Automated Thermal Shading: * Quantity: 5,400 square meters.

  - Spec: Retractable aluminized roof shade-screens.

- Commercial Concrete Slab: * Quantity: 5,400 square meters.

  - Spec: 150mm depth minimum, heavy aquatic point-load reinforcement, sloped to integrated trench drains, epoxy sealed.

- Emergency Diesel Generator: * Quantity: 1 unit.

  - Spec: 400kVA Commercial Generator with Automatic Transfer Switch (ATS) and bulk fuel tank.

- Processing Room Paneling (DAFF Export Grade): * Quantity: Materials for ~150-200 sqm room.

  - Spec: Food-grade PIR coolroom panels, coved corners, high-speed hygienic roller doors.

## 6. STAINLESS STEEL & PROCESSING EQUIPMENT

Vendors: Commercial Kitchen/Abattoir Fitters, Packaging Suppliers

- Cracking/Processing Tables: * Quantity: Custom fit-out.

  - Spec: 316 Grade Stainless Steel benches with integrated waste disposal chutes.

- Live Purging Vats: * Quantity: 4 to 6 units.

  - Spec: 1,000L to 2,000L fiberglass or poly vats.

- Packaging Machines: * Quantity: 1 unit.

  - Spec: Commercial Modified Atmosphere Packaging (MAP) tray sealer.