As a civil engineer looking strictly at the asset creation cost, swapping the high-spec, light-permeable glasshouse structure for a standard, commercial-grade Colorbond/galvanized steel agricultural or industrial shed changes your financial profile considerably.

For a 5,400 square meter footprint at the Australian Bay Lobster Producers (ABLP) site in Chinderah, Northern NSW, we must engineer the structure for Wind Region B (Non-Cyclonic Coast), with a highly reinforced slab capable of carrying massive, localized hydrostatic point-loads (your 170 raceways and 50,000L sumps).

Here is a realistic, 2026-rated civil costing breakdown for a traditional Colorbond industrial shed versus your baseline Glasshouse concept.

### Cost Breakdown: Colorbond/Galvanized Shed Envelope

A commercial "lock-up" warehouse shell or large rural steel shed typically tracks between $350 to $550 per square meter for supply, delivery, and basic installation, but because we are housing a heavy marine aquaculture operation with intensive drainage requirements, the earthworks and slab cannot be compromised.

| Cost Element | Scope & Description | Estimated Cost (AUD) |
| --- | --- | --- |
| Site Prep & Earthworks | Leveling 5,400 sqm, soil compaction, and deep trenching for main plumbing/drainage rough-ins. | $250,000 |
| Engineered Concrete Slab | 150mm commercial-grade, mesh-reinforced slab with epoxy sealant to protect against salt/acid corrosion, integrated sloped trench drains. | $750,000 |
| Primary Structure (Shed Kit) | Galvanized structural steel portal frame, Z purlins, and 0.42+ BMT Colorbond custom-orb roof and wall cladding. | $810,000 (~$150/sqm) |
| Civil Plumbing & Labor | Mains intake, extensive under-slab drainage networks, and commercial pipefitter labor. | $185,000 |
| Erection & Installation Labor | Specialized steel erectors, heavy crane hire, rigging, and structural engineering sign-offs. | $270,000 |
| Subtotal (Base Building Envelope) | Excludes fit-out, internal climate control, or emergency power. | $2,265,000 |
| Contingency (15%) | Civil buffer for coastal water tables/subgrade issues. | $339,750 |
| TOTAL Colorbond Civil Estimate | Fully enclosed, weather-tight building envelope and slab. | $2,604,750 |

### Financial & Operational Comparison

If you move forward with a standard Colorbond portal frame over the original twin-wall polycarbonate/glass structure, you achieve significant upfront capital savings, but it fundamentally shifts your operational model.

- Immediate CAPEX Savings: Your original glasshouse building envelope and civil works totaled $3,691,500 AUD (inclusive of a 15% contingency). Switching to a Colorbond shell drops that specific envelope cost to roughly $2,604,750 AUD. This creates an immediate upfront savings of ~$1,086,750 AUD.

- The Insulation Factor: Standard uninsulated Colorbond turns into an oven during Chinderah's humid sub-tropical summers. To keep the water strictly at 27°C, you would need to add heavy-duty commercial anticon/foil insulation blankets or insulated sandwich panels (such as Bondor) to the ceiling and walls. This would add roughly $25–$50 per sqm ($135,000–$270,000) back into the steel budget.

- The Solar Gain Trade-Off (OPEX Impact): The glasshouse design was chosen as an active thermal trap to subsidize your 180kW heat pump array during the winter, keeping your thermal electricity costs capped at $58,500 annually. In a dark, solid Colorbond shed, you lose all natural solar heating. Your winter heating bills will scale up dramatically because the heat pumps will bear 100% of the load without passive assistance.

- Forfeiting IMTA Future-Proofing: A standard steel shed blocks all natural light. If Mugenuni ever intends to vertically integrate and close the nutrient loop by growing macroalgae (Ulva) indoors, you will have to install millions of dollars worth of high-draw, artificial LED grow-lights and pay the massive ongoing energy bills to power them.

### The Engineering Verdict

From a pure project management and construction cash-flow perspective, building a Colorbond shed on an aquaculture-grade slab will cost you roughly $2.6M AUD (compared to the $3.69M glasshouse envelope).

While it trims over a million dollars from your initial capital raise pitch deck, make sure your financial model accounts for the long-term spike in thermal utility OPEX and the loss of natural lighting flexibility for future expansions.