1. The Physics of UV Contact Time (The "Kill Rate")

UV sterilization relies entirely on exposure time. If you force the entire 450 m³/hr main flow through a UV array, the water is moving so violently fast that the pathogens zip right past the bulbs. To get a lethal dose at that velocity, you would need a massive, incredibly expensive commercial UV reactor array. By using a bypass, you can pull a smaller percentage of water (e.g., 20% to 30% of the main flow), slow it down, and guarantee a 99.9% pathogen kill rate on that side-stream. Because it’s a recirculating system, that continuous "polishing" keeps the overall pathogen load in the whole farm practically at zero.

2. Hydraulic Backpressure (Head Loss)

Forcing your entire main flow through the tight quartz sleeves of a UV bank creates massive backpressure. Your 22kW pumps would have to work significantly harder, burning through your electricity OPEX just to fight the bottleneck.

3. Zero-Downtime Maintenance

UV quartz sleeves get dirty and bulbs burn out (usually around 9,000 hours). If your UV is on a bypass loop, your technicians can simply close two isolation valves, drain the UV vessel, and clean it without ever stopping the main life-support flow to your 170 raceways. If it were flow-through, you would have to shut down the main pumps to service the UV, putting the urchins at risk.

How to integrate it:

Just like we designed a "High-Flow Bypass Manifold" for your Titanium Heat Pumps (costed at $14,200 in the thermal estimate), the UV system will sit on a parallel branch.

The clean water exits the MBBR, the main pumps push it up the trunk line, and a valve bleeds off about 100 m³/hr into the UV/Chiller loops before it all recombines and gravity-feeds out to the raceways.

Does that make sense for the plumbing layout you are envisioning, or were you thinking of using an open-channel gravity UV system?