# Transitioning to the Ulva IMTA Biofiltration System

By moving from a strict Marine RAS to a true IMTA (Integrated Multi-Trophic Aquaculture) system, we are effectively replacing the industrial biofilter with a living, value-generating crop.

## 1. What We Already Have (The Front End)

In our past sessions and your uploaded documents (specifically the Biofiltration and Waste Network CAPEX/OPEX and Drum Filter Optimization files), we have already successfully designed and priced the front end of this flow:

- The Vortex System: We modeled "Twin Radial Flow Settlers" (Swirl/Vortex filters) with 60° cone bottoms to catch the heavy Ulva chaff and intact faecal pellets using gravity.

- The Drum Filter: We modeled a 50-micron automatic drum filter to catch the suspended micro-fines before they dissolve into the water column.

## 2. The Big Change (The Back End)

In the pure RAS model, water flowed from the Drum Filter into a Moving Bed Biofilm Reactor (MBBR) filled with plastic K1 media to process the ammonia.

To put your IMTA hat on, we are now deleting that MBBR from the loop and routing the water from the Drum Filter directly into your Ulva Biofiltration Raceways/Tanks.

## 3. What We Need to Calculate Next

To make this Ulva biofilter work practically and financially in your Chinderah facility plan, we need to map out three new things based on your research (like the Bas de Vos thesis):

- Aeration/Propulsion: As noted in your "Untitled document" upload, Ulva requires a "tumble culture" effect using bottom-mounted air curtains so it doesn't sink and rot. We need to price the blowers for this.

- Spatial Footprint: Ulva raceways take up significantly more floor space than a vertical MBBR tank. We need to see how much of our 5,400 sqm building this will consume.

- Lighting/Solar: Ulva needs high light to absorb the Total Ammonia Nitrogen (TAN). We will leverage your glasshouse design to use free sunlight instead of expensive LEDs.

The Golden Advantage: Instead of just treating waste (an expense), your biofilter will now grow the exact feed you need to re-feed your Tripneustes gratilla (a massive OPEX reduction).