# Ulva IMTA Production: Isolated Unit Economics

## 1. The Strategy: The ABLP Nutrient Interceptor

To produce the 383.25 Tonnes of wet Ulva required annually to feed your 50,000-unit monthly urchin harvest, the urchin waste alone will not provide enough nitrogen/phosphorus.

Instead of buying expensive agricultural fertilizers, Mugenuni will intercept the wastewater pipeline from the neighboring Australian Bay Lobster Producers (ABLP) facility in Chinderah. ABLP currently pipes their effluent to the Kingscliff Sewage Treatment Plant. By diverting a portion of this dirty water through a commercial UV sterilization bank (to ensure absolute biosecurity and prevent lobster pathogens from entering the Ulva system), you gain a free, continuous stream of high-quality marine nutrients.

## 2. Infrastructure Sizing & Design

- Target Yield: 383,250 kg wet Ulva per year.

- Space Required: Assuming a conservative yield of ~90 tonnes of wet Ulva per hectare per year in a controlled environment, we need approximately 4,250 square meters of active water surface area.

- Raceway Dimensions: 17 large-scale raceways, each measuring 50m (L) x 5m (W) x 0.6m (H).

- Construction: Shallow earth-berm trenches lined with heavy-duty marine HDPE geomembrane (vastly cheaper than the Celuka PVC used for the urchins).

- Propulsion: One 0.75kW VFD Paddlewheel per raceway to create the continuous, gentle tumbling flow Ulva requires.

- Housing: Commercial 6m-wide horticulture poly tunnels spanning the raceways to protect from rain dilution while maximizing free solar radiation.

## 3. Isolated CAPEX (Capital Expenditure)

(All figures in AUD)

- Commercial Poly Tunnels (5,000 sqm footprint): * Includes galvanized steel hoops, UV-treated 150-micron poly film, side roll-up ventilation, and installation (~$100/sqm).

  - Cost: $500,000

- HDPE Lined Earth Raceways (17 Units): * Excavation, berm shaping, and 1.5mm HDPE geomembrane liners (~$4,500 per 250 sqm trench).

  - Cost: $76,500

- Propulsion (Paddlewheels): * 17x 0.75kW paddlewheels with VFD controllers @ $750 each.

  - Cost: $12,750

- The ABLP Interceptor & UV Sterilization Bank: * Commercial UV array to sterilize incoming ABLP wastewater, plus tapping valves and transfer pumps.

  - Cost: $35,000

- Harvesting Gear (Nets, Hoists, Carts): * Cost: $8,500

Total Isolated Ulva CAPEX: $632,750 AUD

## 4. Isolated OPEX (Annual Operational Expenditure)

(All figures in AUD)

- Paddlewheel Electricity: * 17 units x 0.75kW running 24/7 = 111,690 kWh/year @ $0.25/kWh.

  - Cost: $27,922

- UV Sterilizer & Transfer Pump Electricity: * Cost: $6,500

- Nutrients / Fertilizer: * Sourced entirely from ABLP wastewater stream & Urchin RAS return.

  - Cost: $0

- Direct Labor (0.6 FTE): * Technician time strictly dedicated to scooping/harvesting the Ulva and moving it to the urchin prep bay.

  - Cost: $45,000

- Routine Maintenance: * UV bulb replacements, poly-film patching, paddlewheel greasing.

  - Cost: $4,500

Total Annual Ulva OPEX: $83,922 AUD

## 5. The Bottom Line: Cost Per Kilogram

By isolating the OPEX and dividing it by your annual production requirement, we find the true cost of producing your primary feed on-site:

- Total Annual OPEX: $83,922

- Total Wet Ulva Produced: 383,250 kg

- Production Cost: $0.21 AUD per kg of wet Ulva

Conclusion for Co-Directors:

Producing our own high-protein Ulva using ABLP’s waste stream and solar poly-tunnels drives our primary feed cost down to 21 cents per wet kilogram. Compared to purchasing wild-harvested macroalgae or manufactured pelleted diets (which often exceed $2.50/kg dry equivalent), this vertically integrated IMTA approach is the single largest operational cost-saver in the Mugenuni business plan.