### Integrated Waste Valorisation for Sea Urchin & Ulva Aquaculture

This document outlines a circular system for managing solid waste within an Integrated Multi-Trophic Aquaculture (IMTA) system growing sea urchins (Tripneustes gratilla) fed on seaweed (Ulva). The process focuses on capturing, fermenting, and reusing waste from both species to enhance sustainability and efficiency.

#### 1. Solids Capture

The initial step is the efficient collection of solid waste, primarily urchin faeces and uneaten or decaying Ulva.

- System Design: Urchins are housed in raceway tanks with 10 mm mesh trays, allowing waste to fall through for collection.

- Primary Filtration: Effluent from the urchin tanks passes through a drum filter to remove larger particles.

- Fine Filtration: A protein skimmer in the sump removes finer suspended solids and dissolved organic waste.

- Manual Collection: Regular cleaning of tanks and sumps supplements automated filtration, collecting accumulated solids and biofouling.

- Ulva Harvesting: Excess Ulva not used for feed is harvested and directed to the digestion system.

#### 2. Anaerobic Digestion (AD)

The collected solids are processed through anaerobic digestion to produce biogas and a nutrient-rich digestate.

- Digester Design: A semi-continuous anaerobic digester is recommended. The feedstock consists of the captured Ulva and urchin waste. Co-digestion with external organic waste can be considered to optimize the carbon-to-nitrogen ratio for biogas production.

- Pre-treatment: Waste is processed to remove grit and is ground into a slurry to prevent blockages and improve degradation.

- Operating Conditions:

  - Temperature: Maintained around 27°C.

  - Solids Retention Time (SRT): Optimal at 5 days for primary sludge and approximately 8.3 days for Ulva co-digestion.

  - pH: Kept between 4.8 and 6.0 to favour the production of valuable volatile fatty acids (VFAs).

  - Mixing: Continuous mixing prevents solids from settling and enhances the degradation process.

- Biogas Management: The digester is covered to capture biogas (primarily methane) for use as a renewable energy source on-site.

#### 3. Digestate Utilisation

The liquid digestate from the AD process is recycled back into the Ulva grow-out system, closing the nutrient loop.

- Nutrient-Rich Output: The digestate is rich in readily available nutrients like dissolved inorganic nitrogen (DIN), phosphorus (P), and VFAs, acting as an effective liquid fertiliser.

- Application to Ulva: The liquid is added to the Ulva paddle raceways. Ulva is an excellent biofilter, rapidly absorbing these nutrients, which enhances its own growth, biomass yield, and protein content.

- Concentration Management: The application of digestate must be controlled to prevent nutrient saturation, ensuring optimal uptake by the Ulva without causing stress.

- Solid Residue: Any remaining solid digestate can be used as a bio-fertiliser for terrestrial agriculture.

#### 4. Benefits and Circularity

This integrated system creates a highly circular and sustainable aquaculture model.

- Waste Valorisation: Transforms waste into valuable resources, including nutrients for Ulva and renewable energy.

- Nutrient Recycling: Reduces the need for external fertilisers and minimises the environmental impact of nutrient discharge.

- Enhanced Production: Boosts Ulva growth and quality, improving it as a feed source for the urchins.

- Economic & Environmental Gains: Creates multiple potential revenue streams (urchins, Ulva, energy, fertiliser), reduces operational costs, and promotes a more eco-friendly aquaculture practice.